CONFIDENTIAL — INTERNAL USE ONLY

REFRIGERATOR NET STORAGE VOLUME CALCULATOR

with Thermodynamic Performance Analysis

Project Brief — Stakeholder Reference
Version 2.0 | May 2026

1. Executive Summary

This project delivers a web-based parametric tool that performs two complementary functions for refrigerator concept design:

- Net storage volume calculation – Derives Gross, Egyptian Net (ES 3794), and IEC Net (IEC 62552) volumes from external cabinet dimensions and internal layout.

- Thermodynamic performance analysis – Solves the steady-state energy balance to predict condensing temperature, evaporator air outlet, compressor running ratio, heat loads, and power consumption.

The thermal analysis is based on a validated Excel model (Sharp SJ-54H) and uses a nested Newton-Raphson solver with refrigerant property correlations.
Both calculators share a common cabinet geometry description, enabling rapid “what-if” comparisons without switching tools.

Target accuracy for volume is within 3% of final CAD values; thermal predictions aim to match the reference Excel workbook to within 1% in converged variables.

2. Project Objectives

- Accept external cabinet dimensions and detailed wall/insulation thicknesses.

- Support up to 8 compartments (fresh food, freezer, convertible) via horizontal/vertical dividers.

- Add internal fittings per compartment (shelves, drawers, door bins, mechanical housings).

- Compute and display Gross, Egyptian Net, and IEC Net volumes simultaneously, in litres and cubic feet.

- Perform a steady-state thermodynamic cycle analysis using a semi-empirical compressor model, refrigerant property correlations, and cabinet heat-load calculations.

- Display thermal results: condensing temperature, evaporator air outlet, running ratio, heat loads, compressor capacity and power.

- Enable saving/loading of configurations (geometry + thermal parameters) as JSON; export both volume and thermal results as CSV.

- Provide a block-diagram schematic of the layout.

3. Calculation Methodology

3.1 Volume Calculation (unchanged)

Three-mode output is calculated simultaneously:

| Mode | Definition | Standard |
| --- | --- | --- |
| Gross Volume | Internal cavity volume, no deductions | None |
| Egyptian Net | Gross minus user-removable accessories | ES 3794 |
| IEC Net | Gross × 0.97 minus all mechanical components | IEC 62552 |

Hierarchy: Gross ≥ EG Net ≥ IEC Net is enforced.

Component deduction rules are as defined in the original brief (Section 3.2).

3.2 Thermodynamic Analysis (New)

The thermal analysis replicates the steady-state model from the Sharp SJ-54H Excel workbook.

Physical model:

- Fixed ambient (T₀), freezer (T_F), refrigerator (T_R) temperatures.

- Fixed evaporating refrigerant temperature (TE = –23.3 °C for R-600a).

- Compressor: EGX80CLC semi-empirical map (volumetric efficiency, mass flow, power).

- Refrigerant properties: curve-fitted correlations for R-600a and R-134a (saturation pressure, specific volume, vapour/liquid enthalpy).

- Cabinet heat loads: calculated from detailed geometry (wall thicknesses, insulation λ), including partition conduction, air-gap losses, and condenser-induced temperature rises on cabinet surfaces.

- Air-side: fan airflow distribution between freezer and refrigerator evaporator.

- Condenser: side/back pipe-coil model with linearised temperature-rise coefficients.

Solver: Nested Newton-Raphson

- Inner loop (2 variables): evaporator air outlet temperature T₂ and compressor running ratio PR.

- Outer loop (1 variable): condensing temperature TC, balancing condenser heat rejection with compressor discharge energy.

Outputs: Converged TC, T₂, PR, QF, QR, QEV, compressor cooling capacity, input power, mass flow, and volumetric efficiency.

Validation: The JavaScript solver has been tested to converge and satisfies the system equations to within 1e-4 tolerance. A direct cross-check against the Excel workbook for exact numerical alignment is pending (see Open Items).

4. Scope

In Scope

- Cabinet external dimensions and per-face wall thicknesses.

- Up to 8 compartments; horizontal/vertical dividers.

- Shelves, drawers, door bins, ice-maker/light housings.

- Three-mode volume output.

- Thermodynamic analysis (steady-state, single-evaporator, fixed-displacement compressor).

- Configuration save/load (JSON) and CSV export.

- Side-by-side comparison of two configurations.

- Block-diagram schematic.

Out of Scope (unchanged, but extended)

- Transient or cyclic thermal analysis.

- Multi-evaporator or variable-speed compressor systems.

- 3D CAD generation, CFD, PLM integration, mobile apps.

- Automatic layout optimization.

5. User Inputs (Expanded)

The user provides inputs via a web interface with two logical sections:

Cabinet Geometry (shared)

- External dimensions: H, W, D (mm)

- Freezer and refrigerator compartment heights (Hf, Hr) – can be derived from compartment layout.

- Bottom heel cutouts (Hb, Db1, Db2)

- Door gap, packing position

- Wall/insulation thicknesses per face for freezer and refrigerator compartments (top, bottom, left, right, rear, door, evaporator back).

Volume Calculator Add-ons

- Compartment layout: number, type, height ratios, vertical splits, fittings as per original brief.

Thermal Parameters (dedicated panel)

- Fixed temperatures: T₀, T_F, T_R (and TE is currently fixed at –23.3 °C)

- Refrigerant selection (R-600a / R-134a)

- Fan total airflow (m³/h)

- Sub-cooling (K), discharge temperature (°C)

- Defrost heater power and ON time

- Compressor parameters (currently pre-loaded; expandable to a library in future).

6. Technical Decisions (Updated)

| Decision | Resolution | Rationale |
| --- | --- | --- |
| Thermal solver | Pure JavaScript, nested Newton-Raphson with Cramer’s rule for 2×2 Jacobian | Runs in browser; no external engine needed. |
| Refrigerant data | Curve-fitted correlations from Excel workbook | Faithful to original model; fast evaluation. |
| Compressor model | Semi-empirical 10-coefficient map | Matches manufacturer data; configurable. |
| Integration | Shared cabinet geometry object; separate config for thermal parameters | Eliminates duplicate input; ensures consistency. |
| Validation | Unit test suite for all thermodynamic functions; solver residual checks; pending Excel cross-check. | Guarantees formula correctness. |

7. Success Criteria (Updated)

- Volume accuracy: within 0.01 L of manual calculation; within ±3% of CAD.

- Thermal accuracy: converged variables (TC, T₂, PR) within ±1% of reference Excel workbook (pending cross-check).

- All three volume modes always satisfy Gross ≥ EG Net ≥ IEC Net.

- Solver consistently converges within 100 inner/outer iterations.

- Configuration comparison functional.

- Reports exportable.

8. Open Items & Pending Decisions

| # | Open Item | Action Required | Blocks |
| --- | --- | --- | --- |
| 1 | Excel cross-check (Phase 1.5) | Manually compare intermediate values (Pc, Pe, v, ηv, heat-loss components) between JS and workbook at converged point. | Final thermal accuracy sign-off. |
| 2 | ES 3794 compliance | Confirm ice maker/light housing classification. | Volume engine finalization. |
| 3 | Compressor library | Add dropdown to select alternative compressors (Vsd, etc.) | Multi-model thermal analysis. |
| 4 | Unified geometry UI | Build single panel feeding both calculators (Phase 2). | User experience consistency. |

9. Deliverables (Expanded)

- Web application with volume calculator and integrated thermodynamic analysis.

- JSON schema for configuration files (includes thermal parameters).

- CSV export of volume results; ability to export thermal summary.

- Unit test suite for calculation engines.

- User manual covering both modules.

- Source code with inline documentation.